Takeuchi TL-10 Maintenance Recommendations

Follow the manufacturer’s service schedule and perform daily checks to keep your Takeuchi TL-10 running efficiently and safely.

Recommended Service Intervals ManualsLib

  - Daily:

    - Grease all grease points

    - Check and adjust crawler belt tension

    - Test operation pedals, steering lever, hourmeter, and warning lamps

    - Check battery fluid level

  - Every 50 hours:

    - Drain fuel tank of water

  - Every 100 hours:

    - Replace fuel filter

  - Every 250 hours:

    - Replace track gearbox oil (1st), oil filter (1st), air cleaner (1st), engine oil (1st)

    - Retighten all bolts

  - Every 500 hours:

    - Replace hydraulic oil (1st), return filter (1st), suction filter (1st), coolant (1st), radiator fins (1st), fan belt (1st)

  - Every 1,000 hours:

    - Repeat 250-hour and 500-hour items (oil, filters, coolant, radiator, fan belt, air cleaner)

Pre-Operation Checks ManualsLib+1

Before starting the TL-10:

  - Check coolant level (30 mm above radiator core when cool; top up with soft water if needed)

  - Adjust fan belt deflection to ~12 mm under ~10 kg pressure

- Check engine oil level (between upper/lower marks)

- Check hydraulic oil level (between marks at normal temp; near upper at operating temp)

- Inspect for oil leaks, damaged bolts/nuts, and electrical component cleanliness